COMPUTER ARCHITECTURE PROJECT REPORT

Part A: Base Assembly Program Students must use the RISC-V assembly program developed in Lab 10 as the starting point. The program must be correctly loaded into the instruction memory of the integrated processor from Lab 11 and executed on hardware. Correct execution must be demonstrated through visible hardware outputs.

TASK A: CODE

`timescale 1ns / 1ps module TopLevelProcessor ( input wire clk, input wire reset_btn, // MATCHES XDC input wire [15:0] sw, // MATCHES XDC output wire [15:0] led, // MATCHES XDC output wire [3:0] an, // NEW output wire [6:0] seg // NEW ); // ========================================== // Internal Signals // ========================================== // Map the XDC ports to your internal logic names here wire rst = reset_btn; wire [15:0] switches = sw; wire [15:0] leds; // assign led = leds; wire [31:0] PC; wire [31:0] PCplus4; wire [31:0] branchTarget; wire [31:0] nextPC; wire PCWrite = 1'b1; // Always 1 for Single-Cycle wire [31:0] instruction; wire [6:0] opcode = instruction[6:0]; wire [4:0] rs1 = instruction[19:15]; wire [4:0] rs2 = instruction[24:20]; wire [4:0] rd = instruction[11:7]; wire [2:0] funct3 = instruction[14:12]; wire isRType = (opcode == 7'b0110011); wire [31:0] readData1, readData2, writeData; wire RegWrite; wire [31:0] ALUInputB, ALUResult; wire ALUZero; wire [3:0] ALUControl; wire [1:0] ALUOp; wire ALUSrc; wire [31:0] dataMemoryRead, dataMemoryWrite, memoryAddress; wire MemRead, MemWrite, Branch, Jump, Jalr; wire [1:0] MemtoReg; wire [31:0] immediate; wire [2:0] immType; // We turn the debug signals into internal wires instead of output ports // so they don't take up physical FPGA pins! wire [31:0] debugPC = PC; wire [31:0] debugInstruction = instruction; wire [31:0] debugALUResult = ALUResult; // --- NEW CLOCK DIVIDER: 100MHz to 1kHz --- reg [25:0] clk_counter = 0; reg slow_clk = 0; always @(posedge clk) begin if (clk_counter == 49999999) begin clk_counter <= 0; slow_clk <= ~slow_clk; end else begin clk_counter <= clk_counter + 1; end end // ----------------------------------------- // ========================================== // Instruction Fetch Stage // ========================================== ProgramCounter pc_unit ( .clk(clk), .rst(rst), // change to slow clock for fpga .nextPC(nextPC), .PCWrite(PCWrite), .PC(PC) ); pcAdder pc_adder ( .PC(PC), .PCplus4(PCplus4) ); instructionMemory #(.OPERAND_LENGTH(31)) instr_mem ( .instAddress(PC), .instruction(instruction) ); // ========================================== // Instruction Decode & Control // ========================================== MainControl main_control ( .opcode(opcode), .RegWrite(RegWrite), .ALUOp(ALUOp), .MemRead(MemRead), .MemWrite(MemWrite), .ALUSrc(ALUSrc), .MemtoReg(MemtoReg), .Branch(Branch), .Jump(Jump), .Jalr(Jalr) ); ALUControl alu_control ( .ALUOp(ALUOp), .funct3(funct3), .funct7_bit(instruction[30] & isRType), // Prevents ADDI from acting as SUB .ALUControlOut(ALUControl) ); assign immType = (opcode == 7'b0100011) ? 3'b001 : // S-type (opcode == 7'b1100011) ? 3'b010 : // B-type (opcode == 7'b1101111) ? 3'b011 : // J-type (JAL) (opcode == 7'b0110111) ? 3'b100 : // NEW: U-type (LUI) 3'b000; // I-type/R-type default immGen imm_gen ( .instruction(instruction), .immType(immType), .immediate(immediate) ); RegisterFile reg_file ( // change to slow clock for fpga .clk(clk), .rst(rst), .WriteEnable(RegWrite), .rs1(rs1), .rs2(rs2), .rd(rd), .WriteData(writeData), .ReadData1(readData1), .ReadData2(readData2) ); // ========================================== // Execute Stage // ========================================== assign ALUInputB = ALUSrc ? immediate : readData2; ALU alu_unit ( .A(readData1), .B(ALUInputB), .ALUControl(ALUControl), .ALUResult(ALUResult), .Zero(ALUZero) ); branchAdder branch_adder ( .PC(PC), .immExtended(immediate), .branchTarget(branchTarget) ); // Evaluate BEQ (000) or BNE (001) wire branchCondition = (funct3 == 3'b000) ? ALUZero : (funct3 == 3'b001) ? ~ALUZero : 1'b0; wire takeBranchOrJump = (Branch & branchCondition) ¦ Jump; wire [31:0] targetPC = Jalr ? {ALUResult[31:1], 1'b0} : branchTarget; mux2 #(.WIDTH(32)) pc_mux ( .in0(PCplus4), .in1(targetPC), .sel(takeBranchOrJump), .out(nextPC) ); // ========================================== // Memory & I/O Stage // ========================================== assign memoryAddress = ALUResult; assign dataMemoryWrite = readData2; wire DataMemSelect, LEDWrite, SwitchReadEnable; AddressDecoder addr_decoder ( .address(memoryAddress), .MemRead(MemRead), .MemWrite(MemWrite), .DataMemSelect(DataMemSelect), .LEDSelect(LEDWrite), .SwitchSelect(SwitchReadEnable) ); DataMemory data_mem ( .clk(clk), // change to slow clock for fpga .MemWrite(MemWrite & DataMemSelect), // Guard against overwriting on LED write .address(memoryAddress), .write_data(dataMemoryWrite), .read_data(dataMemoryRead) ); leds led_interface ( .clk(clk), .rst(rst), // change to slow clock for fpga .writeData(dataMemoryWrite), .writeEnable(LEDWrite), .leds(leds) ); // ========================================== // Write Back Stage // ========================================== wire [31:0] finalReadData; assign finalReadData = SwitchReadEnable ? {16'b0, switches} : dataMemoryRead; // 00: ALU, 01: Mem/IO, 10: PC+4 (For JAL/JALR), 11: Immediate (For LUI) assign writeData = (MemtoReg == 2'b01) ? finalReadData : (MemtoReg == 2'b10) ? PCplus4 : (MemtoReg == 2'b11) ? immediate : // NEW: LUI Path ALUResult; // ========================================== // Output Routing (Normal vs Debug Mode) // ========================================== // Bundle the control signals for Task B demonstration // led[15]=RegWrite, led[14]=MemRead, led[13]=MemWrite, led[12]=Branch, // led[11]=Jump, led[10]=Jalr, led[9]=ALUSrc, led[8:7]=MemtoReg, // led[4]=ALUZero, led[3:0]=ALUControl wire [15:0] debug_signals = { RegWrite, MemRead, MemWrite, Branch, Jump, Jalr, ALUSrc, MemtoReg[1:0], 2'b00, ALUZero, ALUControl[3:0] }; // If sw[15] is UP, show debug flags. If DOWN, show normal LEDs. assign led = sw[15] ? debug_signals : leds; // If sw[15] is UP, show PC on 7-seg. If DOWN, show normal LEDs output. wire [15:0] display_value = sw[15] ? PC[15:0] : leds; // Instantiate the 7-Segment Controller (Uses fast clk) SevenSegmentControl display_unit ( .clk(clk), .rst(rst), .value(display_value), .an(an), .seg(seg) ); endmodule // ============================================================== // Sub-Modules // ============================================================== module ProgramCounter ( input wire clk, input wire rst, input wire [31:0] nextPC, input wire PCWrite, output reg [31:0] PC ); always @(posedge clk) begin if (rst) PC <= 32'b0; else if (PCWrite) PC <= nextPC; end endmodule module pcAdder ( input wire [31:0] PC, output wire [31:0] PCplus4 ); assign PCplus4 = PC + 32'd4; endmodule module branchAdder ( input wire [31:0] PC, input wire [31:0] immExtended, output wire [31:0] branchTarget ); // immExtended already contains the trailing 0 from the immGen. // Do not shift it again! assign branchTarget = PC + immExtended; endmodule module mux2 #(parameter WIDTH = 32) ( input wire [WIDTH-1:0] in0, input wire [WIDTH-1:0] in1, input wire sel, output wire [WIDTH-1:0] out ); assign out = sel ? in1 : in0; endmodule module immGen ( input wire [31:0] instruction, input wire [2:0] immType, // CHANGED: Expanded to 3 bits output reg [31:0] immediate ); wire [11:0] i_imm = instruction[31:20]; wire [6:0] s_hi = instruction[31:25]; wire [4:0] s_lo = instruction[11:7]; wire [12:0] b_raw = {instruction[31], instruction[7], instruction[30:25], instruction[11:8], 1'b0}; wire [20:0] j_raw = {instruction[31], instruction[19:12], instruction[20], instruction[30:21], 1'b0}; // NEW: U-Type extraction wire [31:0] u_imm = {instruction[31:12], 12'b0}; always @(*) begin case (immType) 3'b000: immediate = {{20{i_imm[11]}}, i_imm}; // I-type 3'b001: immediate = {{20{s_hi[6]}}, s_hi, s_lo}; // S-type 3'b010: immediate = {{19{b_raw[12]}}, b_raw}; // B-type 3'b011: immediate = {{11{j_raw[20]}}, j_raw}; // J-type 3'b100: immediate = u_imm; // NEW: U-type (LUI) default: immediate = 32'b0; endcase end endmodule module RegisterFile ( input clk, input rst, input WriteEnable, input [4:0] rs1, input [4:0] rs2, input [4:0] rd, input [31:0] WriteData, output [31:0] ReadData1, output [31:0] ReadData2 ); reg [31:0] regs [31:0]; integer i; always @(posedge clk) begin if (rst) begin regs[0] <= 32'b0; for (i = 1; i < 32; i = i + 1) regs[i] <= 32'b0; end else if (WriteEnable && rd != 5'd0) begin regs[rd] <= WriteData; end end assign ReadData1 = (rs1 == 5'd0) ? 32'b0 : regs[rs1]; assign ReadData2 = (rs2 == 5'd0) ? 32'b0 : regs[rs2]; endmodule module MainControl( input [6:0] opcode, output reg RegWrite, output reg [1:0] ALUOp, output reg MemRead, output reg MemWrite, output reg ALUSrc, output reg [1:0] MemtoReg, output reg Branch, output reg Jump, output reg Jalr ); always @(*) begin RegWrite = 0; ALUOp = 2'b00; MemRead = 0; MemWrite = 0; ALUSrc = 0; MemtoReg = 2'b00; Branch = 0; Jump = 0; Jalr = 0; case(opcode) 7'b0110011: begin RegWrite = 1; ALUOp = 2'b10; end // R-type 7'b0010011: begin RegWrite = 1; ALUSrc = 1; ALUOp = 2'b10; end // I-type 7'b0000011: begin RegWrite = 1; ALUSrc = 1; MemtoReg = 2'b01; MemRead = 1; end // Load 7'b0100011: begin ALUSrc = 1; MemWrite = 1; end // Store 7'b1100011: begin Branch = 1; ALUOp = 2'b01; end // Branch 7'b1101111: begin RegWrite = 1; Jump = 1; MemtoReg = 2'b10; end // JAL 7'b1100111: begin RegWrite = 1; Jump = 1; Jalr = 1; ALUSrc = 1; MemtoReg = 2'b10; end // JALR 7'b0110111: begin RegWrite = 1; MemtoReg = 2'b11; end // NEW: LUI default: ; endcase end endmodule module ALUControl( input [1:0] ALUOp, input [2:0] funct3, input funct7_bit, output reg [3:0] ALUControlOut ); always @(*) begin case(ALUOp) 2'b00: ALUControlOut = 4'b0010; // Ld/St/JALR 2'b01: ALUControlOut = 4'b0110; // Branch 2'b10: begin case(funct3) 3'b000: ALUControlOut = (funct7_bit) ? 4'b0110 : 4'b0010; 3'b111: ALUControlOut = 4'b0000; 3'b110: ALUControlOut = 4'b0001; 3'b100: ALUControlOut = 4'b0011; 3'b001: ALUControlOut = 4'b1000; 3'b101: ALUControlOut = 4'b1001; default: ALUControlOut = 4'b1111; endcase end default: ALUControlOut = 4'b1111; endcase end endmodule module ALU ( input [31:0] A, input [31:0] B, input [3:0] ALUControl, output reg [31:0] ALUResult, output wire Zero ); assign Zero = (ALUResult == 0); always @(*) begin case (ALUControl) 4'b0000: ALUResult = A & B; 4'b0001: ALUResult = A ¦ B; 4'b0010: ALUResult = A + B; 4'b0110: ALUResult = A - B; 4'b0011: ALUResult = A ^ B; 4'b1000: ALUResult = A << B[4:0]; 4'b1001: ALUResult = A >> B[4:0]; default: ALUResult = 32'b0; endcase end endmodule module AddressDecoder ( input [31:0] address, input MemRead, input MemWrite, output DataMemSelect, output LEDSelect, output SwitchSelect ); assign DataMemSelect = ~address[9]; // 0x000 - 0x1FF assign LEDSelect = (address[9:8] == 2'b10) & MemWrite; // 0x200 - 0x2FF (10 in binary is 2) assign SwitchSelect = (address[9:8] == 2'b11) & MemRead; // 0x300 - 0x3FF (11 in binary is 3) endmodule module DataMemory ( input clk, input MemWrite, input [31:0] address, input [31:0] write_data, output [31:0] read_data ); reg [31:0] mem [0:511]; always @(posedge clk) begin // RISC-V uses Byte-Addressing. Word index is address[10:2] if (MemWrite) mem[address[10:2]] <= write_data; end assign read_data = mem[address[10:2]]; endmodule module leds ( input clk, input rst, input [31:0] writeData, input writeEnable, output reg [15:0] leds ); always @(posedge clk) begin if (rst) leds <= 16'b0; else if (writeEnable) leds <= writeData[15:0]; end endmodule module instructionMemory #(parameter OPERAND_LENGTH = 31) ( input [OPERAND_LENGTH:0] instAddress, output reg [31:0] instruction ); reg [7:0] memory [0:255]; always @(*) begin instruction = { memory[instAddress + 3], memory[instAddress + 2], memory[instAddress + 1], memory[instAddress + 0] }; end integer i; // ============= TASK A COUNTDOWN =========== initial begin for (i = 0; i < 256; i = i + 1) memory[i] = 8'h00; // Base memory from your Lab 10 initialization // 0x00: addi x2, x0, 252 # sp = 0xFC memory[ 0] = 8'h13; memory[ 1] = 8'h01; memory[ 2] = 8'hC0; memory[ 3] = 8'h0F; // 0x04: addi x5, x0, 768 # x5 = switch address (0x300) memory[ 4] = 8'h93; memory[ 5] = 8'h02; memory[ 6] = 8'h00; memory[ 7] = 8'h30; // 0x08: addi x6, x0, 512 # x6 = LED address (0x200) memory[ 8] = 8'h13; memory[ 9] = 8'h03; memory[ 10] = 8'h00; memory[ 11] = 8'h20; // 0x0C: sw x0, 0(x6) # clear LEDs on startup memory[ 12] = 8'h23; memory[ 13] = 8'h20; memory[ 14] = 8'h03; memory[ 15] = 8'h00; // 0x10: lw x7, 0(x5) # read switches memory[ 16] = 8'h83; memory[ 17] = 8'hA3; memory[ 18] = 8'h02; memory[ 19] = 8'h00; // 0x14: beq x7, x0, -4 # if 0, loop back to IDLE memory[ 20] = 8'hE3; memory[ 21] = 8'h8E; memory[ 22] = 8'h03; memory[ 23] = 8'hFE; // 0x18: sw x7, 0(x6) # display switch value on LEDs memory[ 24] = 8'h23; memory[ 25] = 8'h20; memory[ 26] = 8'h73; memory[ 27] = 8'h00; // 0x1C: add x10, x7, x0 # x10 = switch value (subroutine arg) memory[ 28] = 8'h33; memory[ 29] = 8'h85; memory[ 30] = 8'h03; memory[ 31] = 8'h00; // 0x20: addi x2, x2, -8 # push stack (caller frame) memory[ 32] = 8'h13; memory[ 33] = 8'h01; memory[ 34] = 8'h81; memory[ 35] = 8'hFF; // 0x24: sw x1, 4(x2) # save ra on stack memory[ 36] = 8'h23; memory[ 37] = 8'h22; memory[ 38] = 8'h11; memory[ 39] = 8'h00; // 0x28: sw x10, 0(x2) # save argument on stack memory[ 40] = 8'h23; memory[ 41] = 8'h20; memory[ 42] = 8'hA1; memory[ 43] = 8'h00; // 0x2C: jal x1, +36 # call COUNTDOWN_SUB at 0x50 memory[ 44] = 8'hEF; memory[ 45] = 8'h00; memory[ 46] = 8'h40; memory[ 47] = 8'h02; // 0x30: lw x1, 4(x2) # restore ra memory[ 48] = 8'h83; memory[ 49] = 8'h20; memory[ 50] = 8'h41; memory[ 51] = 8'h00; // 0x34: addi x2, x2, 8 # pop caller stack frame memory[ 52] = 8'h13; memory[ 53] = 8'h01; memory[ 54] = 8'h81; memory[ 55] = 8'h00; // 0x38: jal x0, +4 # jump to WAIT_RELEASE at 0x3C memory[ 56] = 8'h6F; memory[ 57] = 8'h00; memory[ 58] = 8'h40; memory[ 59] = 8'h00; // WAIT_RELEASE State (PC = 0x3C) // 0x3C: lw x8, 0(x5) # read current switch value memory[ 60] = 8'h03; memory[ 61] = 8'hA4; memory[ 62] = 8'h02; memory[ 63] = 8'h00; // 0x40: bne x8, x0, -4 # if non-zero, loop back to WAIT_RELEASE memory[ 64] = 8'hE3; memory[ 65] = 8'h1E; memory[ 66] = 8'h04; memory[ 67] = 8'hFE; // 0x44: jal x0, -52 # jump back to IDLE at 0x10 memory[ 68] = 8'h6F; memory[ 69] = 8'hF0; memory[ 70] = 8'hDF; memory[ 71] = 8'hFC; // padding memory[ 72] = 8'h13; memory[ 73] = 8'h00; memory[ 74] = 8'h00; memory[ 75] = 8'h00; memory[ 76] = 8'h13; memory[ 77] = 8'h00; memory[ 78] = 8'h00; memory[ 79] = 8'h00; //COUNTDOWN_SUB Subroutine (PC = 0x50) //0x50: addi x2, x2, -16 # allocate 4-word stack frame memory[ 80] = 8'h13; memory[ 81] = 8'h01; memory[ 82] = 8'h01; memory[ 83] = 8'hFF; // 0x54: sw x1, 12(x2) # save ra memory[ 84] = 8'h23; memory[ 85] = 8'h26; memory[ 86] = 8'h11; memory[ 87] = 8'h00; // 0x58: sw x5, 8(x2) # save switch addr memory[ 88] = 8'h23; memory[ 89] = 8'h24; memory[ 90] = 8'h51; memory[ 91] = 8'h00; // 0x5C: sw x6, 4(x2) # save LED addr memory[ 92] = 8'h23; memory[ 93] = 8'h22; memory[ 94] = 8'h61; memory[ 95] = 8'h00; // 0x60: sw x10, 0(x2) # save original count memory[ 96] = 8'h23; memory[ 97] = 8'h20; memory[ 98] = 8'hA1; memory[ 99] = 8'h00; // COUNTDOWN_LOOP (PC = 0x64) // 0x64: sw x10, 0(x6) # leds = current count memory[100] = 8'h23; memory[101] = 8'h20; memory[102] = 8'hA3; memory[103] = 8'h00; // 0x68: beq x10, x0, +24 # if count == 0 jump to COUNTDOWN_DONE memory[104] = 8'h63; memory[105] = 8'h0C; memory[106] = 8'h05; memory[107] = 8'h00; // 0x6C: addi x8, x0, 500 # load delay count memory[108] = 8'h13; memory[109] = 8'h04; memory[110] = 8'h20; memory[111] = 8'h00; // DELAY_LOOP (PC = 0x70) // 0x70: addi x8, x8, -1 # decrement delay counter memory[112] = 8'h13; memory[113] = 8'h04; memory[114] = 8'hF4; memory[115] = 8'hFF; // 0x74: bne x8, x0, -4 # loop back to DELAY_LOOP memory[116] = 8'hE3; memory[117] = 8'h1E; memory[118] = 8'h04; memory[119] = 8'hFE; // 0x78: addi x10, x10, -1 # count-- memory[120] = 8'h13; memory[121] = 8'h05; memory[122] = 8'hF5; memory[123] = 8'hFF; // 0x7C: jal x0, -24 # jump back to COUNTDOWN_LOOP memory[124] = 8'h6F; memory[125] = 8'hF0; memory[126] = 8'h9F; memory[127] = 8'hFE; // COUNTDOWN_DONE (PC = 0x80) // 0x80: sw x0, 0(x6) # clear LEDs memory[128] = 8'h23; memory[129] = 8'h20; memory[130] = 8'h03; memory[131] = 8'h00; // 0x84: lw x10, 0(x2) # restore original count memory[132] = 8'h03; memory[133] = 8'h25; memory[134] = 8'h01; memory[135] = 8'h00; // 0x88: lw x6, 4(x2) # restore LED address memory[136] = 8'h03; memory[137] = 8'h23; memory[138] = 8'h41; memory[139] = 8'h00; // 0x8C: lw x5, 8(x2) # restore switch address memory[140] = 8'h83; memory[141] = 8'h22; memory[142] = 8'h81; memory[143] = 8'h00; // 0x90: lw x1, 12(x2) # restore ra memory[144] = 8'h83; memory[145] = 8'h20; memory[146] = 8'hC1; memory[147] = 8'h00; // 0x94: addi x2, x2, 16 # free subroutine stack frame memory[148] = 8'h13; memory[149] = 8'h01; memory[150] = 8'h01; memory[151] = 8'h01; // 0x98: jalr x0, x1, 0 # return to caller memory[152] = 8'h67; memory[153] = 8'h80; memory[154] = 8'h00; memory[155] = 8'h00; end endmodule

TASK A: TEST BENCH

`timescale 1ns / 1ps module tb_ProjectFinal; // Inputs reg clk; reg reset_btn; reg [15:0] sw; // Outputs wire [15:0] led; wire [3:0] an; wire [6:0] seg; // Instantiate the Unit Under Test (UUT) TopLevelProcessor uut ( .clk(clk), .reset_btn(reset_btn), .sw(sw), .led(led), .an(an), .seg(seg) ); // 100MHz Clock Generation always #5 clk = ~clk; initial begin // Setup Waveforms $dumpfile("tb_ProjectFinal.vcd"); $dumpvars(0, tb_ProjectFinal); // 1. Initialize System (Switches OFF) clk = 0; reset_btn = 1; sw = 16'h0000; // 2. Release Reset #20; reset_btn = 0; $display("Processor started. Waiting in IDLE loop..."); // Let the processor run a few cycles to hit the WAIT_RELEASE / IDLE state #100; // 3. Trigger the Countdown // We set the switches to 3. The processor should read this, // jump to COUNTDOWN_SUB, and start counting down on the LEDs. $display("Flipping switches to 6..."); sw = 16'h0006; // 4. Let it run! // Because your program has a delay loop (addi x8, x0, 500) between counts, // we need to give the simulation enough time to finish the nested loops. #2000000; $display("Simulation time finished."); $finish; end // Real-time LED Monitor // This will print to the TCL console exactly when the LEDs update, // so you don't even have to look at the waveforms if you don't want to! always @(led) begin $display("Time: %0t ns ¦ LEDs changed to: %h", $time, led); end // Real-time Control Signal Monitor // This peers inside your top module to print the exact flags always @(posedge clk) begin if (!reset_btn) begin $display("Time: %0t ns ¦ PC: %h ¦ MemtoReg: %b ¦ Jump: %b ¦ Jalr: %b", $time, uut.PC, uut.MemtoReg, uut.Jump, uut.Jalr); end end endmodule

TASK A: SIMULATION

TCL CONSOLE

Time resolution is 1 ps Time: 5000 ns ¦ LEDs changed to: 0000 Processor started. Waiting in IDLE loop... Time: 25000 ns ¦ PC: 00000000 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 35000 ns ¦ PC: 00000004 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 45000 ns ¦ PC: 00000008 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 55000 ns ¦ PC: 0000000c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 65000 ns ¦ PC: 00000010 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 75000 ns ¦ PC: 00000014 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 85000 ns ¦ PC: 00000010 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 95000 ns ¦ PC: 00000014 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 105000 ns ¦ PC: 00000010 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 115000 ns ¦ PC: 00000014 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Flipping switches to 6... Time: 125000 ns ¦ PC: 00000010 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 135000 ns ¦ PC: 00000014 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 145000 ns ¦ PC: 00000018 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 145000 ns ¦ LEDs changed to: 0006 Time: 155000 ns ¦ PC: 0000001c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 165000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 175000 ns ¦ PC: 00000024 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 185000 ns ¦ PC: 00000028 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 195000 ns ¦ PC: 0000002c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 205000 ns ¦ PC: 00000050 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 215000 ns ¦ PC: 00000054 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 225000 ns ¦ PC: 00000058 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 235000 ns ¦ PC: 0000005c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 245000 ns ¦ PC: 00000060 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 255000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 265000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 275000 ns ¦ PC: 0000006c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 285000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 295000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 305000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 315000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 325000 ns ¦ PC: 00000078 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 335000 ns ¦ PC: 0000007c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 345000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 345000 ns ¦ LEDs changed to: 0005 Time: 355000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 365000 ns ¦ PC: 0000006c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 375000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 385000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 395000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 405000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 415000 ns ¦ PC: 00000078 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 425000 ns ¦ PC: 0000007c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 435000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 435000 ns ¦ LEDs changed to: 0004 Time: 445000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 455000 ns ¦ PC: 0000006c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 465000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 475000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 485000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 495000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 505000 ns ¦ PC: 00000078 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 515000 ns ¦ PC: 0000007c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 525000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 525000 ns ¦ LEDs changed to: 0003 Time: 535000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 545000 ns ¦ PC: 0000006c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 555000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 565000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 575000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 585000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 595000 ns ¦ PC: 00000078 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 605000 ns ¦ PC: 0000007c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 615000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 615000 ns ¦ LEDs changed to: 0002 Time: 625000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 635000 ns ¦ PC: 0000006c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 645000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 655000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 665000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 675000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 685000 ns ¦ PC: 00000078 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 695000 ns ¦ PC: 0000007c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 705000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 705000 ns ¦ LEDs changed to: 0001 Time: 715000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 725000 ns ¦ PC: 0000006c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 735000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 745000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 755000 ns ¦ PC: 00000070 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 765000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 775000 ns ¦ PC: 00000078 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 785000 ns ¦ PC: 0000007c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 795000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 795000 ns ¦ LEDs changed to: 0000 Time: 805000 ns ¦ PC: 00000068 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 815000 ns ¦ PC: 00000080 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 825000 ns ¦ PC: 00000084 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 835000 ns ¦ PC: 00000088 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 845000 ns ¦ PC: 0000008c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 855000 ns ¦ PC: 00000090 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 865000 ns ¦ PC: 00000094 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 875000 ns ¦ PC: 00000098 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 1 Time: 885000 ns ¦ PC: 00000030 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 895000 ns ¦ PC: 00000034 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 905000 ns ¦ PC: 00000038 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 915000 ns ¦ PC: 0000003c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 925000 ns ¦ PC: 00000040 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 935000 ns ¦ PC: 0000003c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 945000 ns ¦ PC: 00000040 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 955000 ns ¦ PC: 0000003c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 965000 ns ¦ PC: 00000040 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 975000 ns ¦ PC: 0000003c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 985000 ns ¦ PC: 00000040 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 995000 ns ¦ PC: 0000003c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 relaunch_sim: Time (s): cpu = 00:00:00 ; elapsed = 00:00:08 . Memory (MB): peak = 1417.719 ; gain = 0.000

HARDWARE ON FPGA

ANALYSIS

### . Functional Verification of Instruction Fetch & Decode The simulation confirms that the Program Counter (PC) correctly iterates through the instruction memory. Initially, the processor resides in an IDLE state (polling loop), evidenced by the PC toggling between 0x10 and 0x14. This validates the implementation of the lw instruction for reading switches and the beq instruction for conditional branching. ### 2. Subroutine Control Flow and Stack Management A critical aspect of Part A is the transition from the main loop to the COUNTDOWN_SUB subroutine. At approximately 195,000 ns, the simulation captures a successful jump triggered by the jal instruction at 0x2C. - Jump Verification: The Jump control signal is asserted, and the branchTarget is correctly calculated as 0x50. - Link Verification: The return address (PC+4 = 0x30) is stored in the link register (x1). - Stack Integrity: The simulation shows the Stack Pointer (x2) decrementing to allocate a 4-word frame. This proves that the RegisterFile and DataMemory are correctly synchronized to preserve the caller’s state during the countdown. ### 3. Execution of Nested Loops and Delay Logic The "Heartbeat" of the simulation is the delay loop execution. The PC spend the majority of its cycles toggling between 0x70 and 0x74. - Decrement Logic: The ALU correctly performs the subtraction (addi x8, x8, -1). - Branch Accuracy: The bne instruction at 0x74 accurately evaluates the Zero flag from the ALU, ensuring the delay loop executes exactly the number of times specified in the machine code. ### 4. Memory-Mapped I/O and Peripheral Interfacing The simulation proves the effectiveness of the AddressDecoder. When the countdown value in register x10 is updated: - The sw (Store Word) instruction targets address 0x200. - The AddressDecoder asserts the LEDWrite signal. - The leds interface module updates the led output bus. As seen in the waveform, the led signal successfully transitions from 0006 through 0000, confirming that the software-to-hardware interface is seamless. ### 5. Program Termination and Context Restore Upon reaching the terminal count of 0000, the branch at 0x68 (beq x10, x0, +24) is taken. The subsequent simulation cycles show the restoration of registers from the stack and a successful return to the caller via the jalr instruction at 0x98. The return to PC 0x30 confirms the complete "Fetch-Decode-Execute-Return" cycle for subroutines is robust.

Part B: Instruction Extension Students are required to implement any three (3) RISC-V instructions that are not already included and are of different types in the provided processor code. Each selected instruction must be properly verified using assembly-level testing. All added instructions must be demonstrated on hardware.

TASK B: CODE

// TASK B initial begin for (i = 0; i < 256; i = i + 1) memory[i] = 8'h00; // 0x00: lui x5, 0x0000A (MemtoReg = 11) memory[0] = 8'h37; memory[1] = 8'hA2; memory[2] = 8'h00; memory[3] = 8'h00; // 0x04: jal x1, 8 (Jump to subroutine at 0x0C. Jump flag = 1) memory[4] = 8'hEF; memory[5] = 8'h00; memory[6] = 8'h80; memory[7] = 8'h00; // 0x08: jal x0, -8 (Jump back to main at 0x00) memory[8] = 8'h6F; memory[9] = 8'hF0; memory[10] = 8'h9F; memory[11] = 8'hFF; // --- SUBROUTINE --- // 0x0C: jalr x0, x1, 0 (Jump back to caller at 0x08. Jalr flag = 1) memory[12] = 8'h67; memory[13] = 8'h80; memory[14] = 8'h00; memory[15] = 8'h00; end

TASK B: TCL CONSOLE

Time resolution is 1 ps Time: 0 ns ¦ LEDs changed to: xxXx Time: 5000 ns ¦ LEDs changed to: xxXx Time: 5000 ns ¦ LEDs changed to: 819f Time: 5000 ns ¦ LEDs changed to: 8192 Processor started. Waiting in IDLE loop... ----------------------------------------------------- Task B Simulation: Verifying LUI, JAL, and JALR ----------------------------------------------------- Time: 25000 ns ¦ PC: 00000000 ¦ MemtoReg: 11 ¦ Jump: 0 ¦ Jalr: 0 Time: 25000 ns ¦ LEDs changed to: 8912 Time: 35000 ns ¦ PC: 00000004 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 35000 ns ¦ LEDs changed to: 8902 Time: 35000 ns ¦ LEDs changed to: 8f02 Time: 45000 ns ¦ PC: 0000000c ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 1 Time: 45000 ns ¦ LEDs changed to: 8f12 Time: 45000 ns ¦ LEDs changed to: 8912 Time: 55000 ns ¦ PC: 00000008 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 55000 ns ¦ LEDs changed to: 8902 Time: 55000 ns ¦ LEDs changed to: 8182 Time: 65000 ns ¦ PC: 00000000 ¦ MemtoReg: 11 ¦ Jump: 0 ¦ Jalr: 0 Time: 65000 ns ¦ LEDs changed to: 8192 Time: 65000 ns ¦ LEDs changed to: 8912 Time: 75000 ns ¦ PC: 00000004 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 75000 ns ¦ LEDs changed to: 8902 Time: 75000 ns ¦ LEDs changed to: 8f02 $finish called at time : 80 ns : File "E:/CA Lab/CA Lab Project/CA Lab Project.srcs/sim_1/new/tb_ProjectFinal.v" Line 86 relaunch_sim: Time (s): cpu = 00:00:00 ; elapsed = 00:00:08 . Memory (MB): peak = 1848.352 ; gain = 0.000

TASK B: TESTBENCH

// TASK B initial begin // 1. Initialize with Switch 15 ON (Debug Mode) clk = 0; reset_btn = 1; sw = 16'h8000; // 1000_0000_0000_0000 in binary // 2. Release Reset #20; reset_btn = 0; $display("-----------------------------------------------------"); $display("Task B Simulation: Verifying LUI, JAL, and JALR"); $display("-----------------------------------------------------"); // 3. Run for 60 nanoseconds (just enough to see the loop execute once) #60; $finish; end

TASK B: SIMULATION

TASK B: ANALYSIS

The table below summarises the expected control signal values for each instruction in the Task B sequence, cross-referenced against what was observed on the LED display during hardware execution: PC; Instruction; RegWrite; ALUSrc; MemtoReg; Jump; JALR; ALUControl. 0x0000; lui x5, 0xA; 1; 0; 11; 0; 0; 0010. 0x0004; jal x1, 8; 1; 0; 10; 1; 0; 0010. 0x0008; jalr x0, x1; 0; 1; 10; 1; 1; 0010.

TASK B: HARDWARE ON FPGA

Part C: Program Demonstration on Hardware In addition to the base program and instruction extensions, students must demonstrate at least one meaningful assembly-level program executing on the processor. Examples include, but are not limited to: • Fibonacci series computation • Factorial calculation • Summation of an array • Loop-based arithmetic sequence The program output must be observable on hardware, and students must be able to explain the execution flow and results.

TASK C: CODE

//FIBONCACCI SERIES initial begin for (i = 0; i < 256; i = i + 1) memory[i] = 8'h00; // --- SETUP --- // 0x00: addi sp, zero, 508 memory[ 0] = 8'h13; memory[ 1] = 8'h01; memory[ 2] = 8'hC0; memory[ 3] = 8'h1F; // 0x04: addi x26, zero, 768 (s10 = Switches) memory[ 4] = 8'h13; memory[ 5] = 8'h0D; memory[ 6] = 8'h00; memory[ 7] = 8'h30; // 0x08: addi x27, zero, 512 (s11 = LEDs) memory[ 8] = 8'h93; memory[ 9] = 8'h0D; memory[ 10] = 8'h00; memory[ 11] = 8'h20; // --- MAIN_LOOP --- // 0x0C: lw x10, 0(x26) memory[ 12] = 8'h03; memory[ 13] = 8'h25; memory[ 14] = 8'h0D; memory[ 15] = 8'h00; // 0x10: beq x10, zero, -4 memory[ 16] = 8'hE3; memory[ 17] = 8'h0E; memory[ 18] = 8'h05; memory[ 19] = 8'hFE; // 0x14: jal x1, 24 (Call fib at 0x2C) memory[ 20] = 8'hEF; memory[ 21] = 8'h00; memory[ 22] = 8'h80; memory[ 23] = 8'h01; // 0x18: sw x10, 0(x27) memory[ 24] = 8'h23; memory[ 25] = 8'hA0; memory[ 26] = 8'hAD; memory[ 27] = 8'h00; // --- WAIT_RESET --- // 0x1C: lw x5, 0(x26) memory[ 28] = 8'h83; memory[ 29] = 8'h22; memory[ 30] = 8'h0D; memory[ 31] = 8'h00; // 0x20: bne x5, zero, -4 memory[ 32] = 8'hE3; memory[ 33] = 8'h9E; memory[ 34] = 8'h02; memory[ 35] = 8'hFE; // 0x24: sw zero, 0(x27) memory[ 36] = 8'h23; memory[ 37] = 8'hA0; memory[ 38] = 8'h0D; memory[ 39] = 8'h00; // 0x28: jal zero, -28 (Jump to 0x0C) memory[ 40] = 8'h6F; memory[ 41] = 8'hF0; memory[ 42] = 8'h5F; memory[ 43] = 8'hFE; // --- SUBROUTINE: FIB --- // 0x2C: addi sp, sp, -12 memory[ 44] = 8'h13; memory[ 45] = 8'h01; memory[ 46] = 8'h41; memory[ 47] = 8'hFF; // 0x30: sw x1, 8(sp) memory[ 48] = 8'h23; memory[ 49] = 8'h24; memory[ 50] = 8'h11; memory[ 51] = 8'h00; // 0x34: sw x8, 4(sp) memory[ 52] = 8'h23; memory[ 53] = 8'h22; memory[ 54] = 8'h81; memory[ 55] = 8'h00; // 0x38: sw x9, 0(sp) memory[ 56] = 8'h23; memory[ 57] = 8'h20; memory[ 58] = 8'h91; memory[ 59] = 8'h00; // 0x3C: addi x5, zero, 1 memory[ 60] = 8'h93; memory[ 61] = 8'h02; memory[ 62] = 8'h10; memory[ 63] = 8'h00; // 0x40: beq x10, x5, 40 (Jump to 0x68 fib_done) memory[ 64] = 8'h63; memory[ 65] = 8'h04; memory[ 66] = 8'h55; memory[ 67] = 8'h02; // 0x44: addi x8, zero, 0 memory[ 68] = 8'h13; memory[ 69] = 8'h04; memory[ 70] = 8'h00; memory[ 71] = 8'h00; // 0x48: addi x9, zero, 1 memory[ 72] = 8'h93; memory[ 73] = 8'h04; memory[ 74] = 8'h10; memory[ 75] = 8'h00; // 0x4C: addi x10, x10, -1 memory[ 76] = 8'h13; memory[ 77] = 8'h05; memory[ 78] = 8'hF5; memory[ 79] = 8'hFF; // --- FIB_LOOP --- // 0x50: add x5, x8, x9 memory[ 80] = 8'hB3; memory[ 81] = 8'h02; memory[ 82] = 8'h94; memory[ 83] = 8'h00; // 0x54: addi x8, x9, 0 memory[ 84] = 8'h13; memory[ 85] = 8'h84; memory[ 86] = 8'h04; memory[ 87] = 8'h00; // 0x58: addi x9, x5, 0 memory[ 88] = 8'h93; memory[ 89] = 8'h84; memory[ 90] = 8'h02; memory[ 91] = 8'h00; // 0x5C: addi x10, x10, -1 memory[ 92] = 8'h13; memory[ 93] = 8'h05; memory[ 94] = 8'hF5; memory[ 95] = 8'hFF; // 0x60: bne x10, zero, -16 (Jump to 0x50) memory[ 96] = 8'hE3; memory[ 97] = 8'h18; memory[ 98] = 8'h05; memory[ 99] = 8'hFE; // 0x64: addi x10, x9, 0 memory[100] = 8'h13; memory[101] = 8'h85; memory[102] = 8'h04; memory[103] = 8'h00; // --- FIB_DONE --- // 0x68: lw x9, 0(sp) memory[104] = 8'h83; memory[105] = 8'h24; memory[106] = 8'h01; memory[107] = 8'h00; // 0x6C: lw x8, 4(sp) memory[108] = 8'h03; memory[109] = 8'h24; memory[110] = 8'h41; memory[111] = 8'h00; // 0x70: lw x1, 8(sp) memory[112] = 8'h83; memory[113] = 8'h20; memory[114] = 8'h81; memory[115] = 8'h00; // 0x74: addi sp, sp, 12 memory[116] = 8'h13; memory[117] = 8'h01; memory[118] = 8'hC1; memory[119] = 8'h00; // 0x78: jalr zero, x1, 0 memory[120] = 8'h67; memory[121] = 8'h80; memory[122] = 8'h00; memory[123] = 8'h00; end

TASK C: SIMULATION

TASK C: TCL CONSOLE

Time resolution is 1 ps Time: 5000 ns ¦ LEDs changed to: 0000 Processor started. Waiting in IDLE loop... Time: 25000 ns ¦ PC: 00000000 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 35000 ns ¦ PC: 00000004 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 45000 ns ¦ PC: 00000008 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 55000 ns ¦ PC: 0000000c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 65000 ns ¦ PC: 00000010 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 75000 ns ¦ PC: 0000000c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 85000 ns ¦ PC: 00000010 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 95000 ns ¦ PC: 0000000c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 105000 ns ¦ PC: 00000010 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 115000 ns ¦ PC: 0000000c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Flipping switches to 6... Time: 125000 ns ¦ PC: 00000010 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 135000 ns ¦ PC: 0000000c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 145000 ns ¦ PC: 00000010 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 155000 ns ¦ PC: 00000014 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 0 Time: 165000 ns ¦ PC: 0000002c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 175000 ns ¦ PC: 00000030 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 185000 ns ¦ PC: 00000034 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 195000 ns ¦ PC: 00000038 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 205000 ns ¦ PC: 0000003c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 215000 ns ¦ PC: 00000040 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 225000 ns ¦ PC: 00000044 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 235000 ns ¦ PC: 00000048 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 245000 ns ¦ PC: 0000004c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 255000 ns ¦ PC: 00000050 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 265000 ns ¦ PC: 00000054 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 275000 ns ¦ PC: 00000058 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 285000 ns ¦ PC: 0000005c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 295000 ns ¦ PC: 00000060 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 305000 ns ¦ PC: 00000050 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 315000 ns ¦ PC: 00000054 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 325000 ns ¦ PC: 00000058 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 335000 ns ¦ PC: 0000005c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 345000 ns ¦ PC: 00000060 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 355000 ns ¦ PC: 00000050 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 365000 ns ¦ PC: 00000054 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 375000 ns ¦ PC: 00000058 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 385000 ns ¦ PC: 0000005c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 395000 ns ¦ PC: 00000060 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 405000 ns ¦ PC: 00000050 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 415000 ns ¦ PC: 00000054 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 425000 ns ¦ PC: 00000058 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 435000 ns ¦ PC: 0000005c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 445000 ns ¦ PC: 00000060 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 455000 ns ¦ PC: 00000050 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 465000 ns ¦ PC: 00000054 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 475000 ns ¦ PC: 00000058 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 485000 ns ¦ PC: 0000005c ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 495000 ns ¦ PC: 00000060 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 505000 ns ¦ PC: 00000064 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 515000 ns ¦ PC: 00000068 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 525000 ns ¦ PC: 0000006c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 535000 ns ¦ PC: 00000070 ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 545000 ns ¦ PC: 00000074 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 555000 ns ¦ PC: 00000078 ¦ MemtoReg: 10 ¦ Jump: 1 ¦ Jalr: 1 Time: 565000 ns ¦ PC: 00000018 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 565000 ns ¦ LEDs changed to: 0008 Time: 575000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 585000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 595000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 605000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 615000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 625000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 635000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 645000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 655000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 665000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 675000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 685000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 695000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 705000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 715000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 725000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 735000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 745000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 755000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 765000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 775000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 785000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 795000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 805000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 815000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 825000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 835000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 845000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 855000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 865000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 875000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 885000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 895000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 905000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 915000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 925000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 935000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 945000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 955000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 965000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 975000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 Time: 985000 ns ¦ PC: 00000020 ¦ MemtoReg: 00 ¦ Jump: 0 ¦ Jalr: 0 Time: 995000 ns ¦ PC: 0000001c ¦ MemtoReg: 01 ¦ Jump: 0 ¦ Jalr: 0 relaunch_sim: Time (s): cpu = 00:00:01 ; elapsed = 00:00:08 . Memory (MB): peak = 1848.352 ; gain = 0.000

TASK C: ANALYSIS

TASK C: HARDWARE ON FPGA

SW = 8 → Fib(8) = 21 = 0x15. LEDs show 0b10101 (binary), 7-segment shows 0x0015 (for n=6) SW = 8 → Fib(8) = 21 = 0x15. LEDs show 0b10101 (binary), 7-segment shows 0x0015